Tiny Tapeout Project – AXI Stream to UART Communication System

Overview:

- The AXI Stream to UART system transfers parallel data from an AXI-Stream interface to a UART serial channel.

- Data send by the AXI-Stream master is pushed into a synchronous FIFO buffer. The FIFO output is then serialized and transmitted by the UART transmitter.

- On the receiver side, the UART receiver deserializes the data, checks parity, and outputs parallel data.

Block Diagram:

Functionality:

| Module | Function |
| --- | --- |
| AXIS Master Input | sends valid data packets for transmission using AXI Stream handshaking signals (valid, ready, last). |
| Synchronous FIFO | Buffers AXI Stream data to handle speed differences between producer (AXIS Master) and consumer (UART TX). Supports parallel read/write operations and “full/empty” detection. |
| UART Transmitter | Converts parallel FIFO data into serial format as per configured baud rate. Supports even parity generation. |
| UART Receiver | Deserializes received UART stream, reconstructs parallel data, verifies parity, and asserts rx_valid when data is ready and parity is verified. |

Pins:

| Pin Name | Direction | Width | Function |
| --- | --- | --- | --- |
| clk | Input | 1 | Clock Signal |
| rst | Input | 1 | Active High Synchronous Reset |
| axis_data | Input | 8 | Parallel input data from AXI Stream master |
| axis_valid | Input | 1 | AXI Stream valid signal – indicates valid data for axis_data |
| axis_last | Input | 1 | Indicates last data in a frame |
| m_axis_ready | Output | 1 | Indicates that system is ready to accept new AXI data i.e. the FIFO is not full. |
| uart_tx | Output | 1 | UART serial transmit line |
| rx_valid | Output | 1 | High for one clock cycle when new data is received successfully |
| rx_data | Output | 8 | Parallel data after UART reception |

Operating Sequence:

- AXI Input Stage: AXI master asserts axis_valid with data on axis_data. When m_axis_ready is high, data is accepted and written to FIFO (axis_last is asserted after the last byte is sent with 1 clock cycle delay).

- FIFO Buffering: FIFO stores incoming bytes until the UART transmitter is ready to send. Read and write can occur simultaneously.

- UART Transmission:

- Each byte is serialized at 115200 baud.

- Start bit (0), data bits (8), parity bit, and stop bit (1) are sent.

- tx_data_ready indicates when UART can accept new data.

- UART Reception:

- Receiver detects start bit, samples bits at mid-baud intervals.

- Extracts data, verifies parity, and asserts rx_valid.

Design Parameters:

| Parameter | Value |
| --- | --- |
| Data Bits | 8 bits per UART frame |
| Baud Rate | 115200 |
| Clock Frequency | 50 Mhz |
| FIFO Depth | 8 bytes |

Other Details:

- AXI Stream Protocol – The data transfer takes place only when VALID(axis_valid) = 1 and READY(m_axis_ready) = 1.

Start Bit

Stop Bit

Parity Bit

Data Bits

UART Frame Format –

AXI stream protocol channels

M_valid :indicates valid input data for axis_data.

M_data : 8 bit input parallel data.

M_last: indicate last byte of the input data.

S_ready : signal used for handshake indicates that system is ready to accept new AXI     data.